Original research paper; UDK 641.1:338.439.5:339.133.2/.3. DOI: xxxxxxxxxxxxxx.

FATTY ACID COMPOSITION OF VARIOUS SOYBEAN PRODUCTS

Dušica S. Ivanov*1, Jovanka D. Lević1, Slavica A. Sredanović1

1University of Novi Sad, Institute of Food Technology, 21000 Novi Sad,

Bulevar cara Lazara 1, Serbia

Abstract: The use of soybean products in feed and food industry has increased steadily over the past decade. According to its end uses, it can be classified as oil bean and food bean. A wide range of processes is known today, in aim to produce large variety of soybean products for human and animal nutrition. The aim of the study presented in this paper was to investigate fatty acid composition in soybean and various soybean products, and to get overall picture about their nutritional suitability, regarding fatty acid composition. Whole bean of soy, soybean oil, soybean cake and soybean grits were investigated. Total fat content was extracted from the samples by supercritical fluid extraction with CO2, and fatty acid profile was determined on gas chromatograph equipped with flame ionization detector. Most prevalent fatty acids in soybean and soybean products, except in soybean oil, were C18:2 ω-6, C18:1, and C18:3 ω-3. All examined samples have had PUFA/SFA ratio higher than 0.4, and thus, soybean and its products can be considered as favorable. Examined soybean and soybean products have had ω–6/ω–3 ratio higher than 4 (preferred ratio is less than 4), and therefore cannot be used as the only source of fatty acids in human and animal nutrition. Key words: ω-3 fatty acid, ω-6 fatty acid, soybean, full-fat grits, cake, oil

Introduction

Soybean (Glycine max (L.) Merrill) originated from China, where it is considered the oldest agricultural crop. It is known in Europe for over 200 years. A large number of soybean varieties exist, producing soybeans that vary greatly in shape and colour (Karlović & Andrić, 1996). The soybean consist of two cotyledons which represent approximately 90% of the weight, a seed coat of hull (8% of weight), and two much smaller and lighter structures, the hypocotyls and the plumose (Van Eys, Offner & Bach, 2004). The mature soybean is about 38% protein, 30% carbohydrate, 18% oil, and 14% moisture, ash, and hull. Soybeans contain all three of the macronutrients required for good nutrition: complete protein, carbohydrate and fat, as well as vitamins and minerals, including calcium, folic acid, and iron (Sauvant, Perez & Tran, 2004).

The use of soybean products in feed and food industry has increased steadily over the past decade. Soybean is especially interesting because of its non-gluten proteins (Torbica, Hadnađev, Dokić & Sakač, 2008). A large array of different manufacturing processes is applied to obtain the many soy products used in animal and human nutrition (Berk, 1992). Soybean can be classified into oil bean and food bean according to its end uses. Oil soybean, i.e. commodity bean, is the primary source of vegetable oil and soy protein products, such as defatted soy flour and soy protein concentrate (Van Eys et al., 2004). Graphic illustration of process production of various products is shown in Figure 1 (Van Eys et al., 2004). Soybean cake is the product gained from pressing of soybean, with 15% of proteins soluble in water. Soybean grit is the meal product resulting from extrusion of whole soybeans without removing any of the component parts (Pravilnik o kvalitetu hrane za životinje, 2009).

Recently, there has been growing interest for fatty acid composition of food, since there has been reported a great influence of these components on human health. The beneficial health effects of ω-3 fatty acids were described first in the Greenland Eskimos (Rose et al., 1999) who consumed a diet rich in seafood and had low rates of coronary heart disease, asthma, type 1 diabetes mellitus, and multiple sclerosis, since that observation, the beneficial effects of ω-3 fatty acids have been extended to include benefits related to cancer, inflammatory bowel disease, rheumatoid arthritis, and psoriasis (Rose & Connoly, 1999; Connor, 2000; Simopoulos, 2002; Simopoulos, 2008). At the same time increased levels of ω-6 fatty acids are associated with an increase in chronic diseases (Givens, Kliem & Gibbs, 2006).

However, these beneficial, health promoting effects are limited by the fact that the modern Western diet is rather low in ω-3 fatty acids and is very high in ω-6 fatty acids content (Enser, Richardson, Wood, Gill & Sheard, 2000).

The aim of the study presented in this paper was to investigate fatty acid composition in soybean and various soybean products, and to obtain overall picture about their nutritional suitability, from the point of fatty acids.

Figure 1. Schematic presentation of the manufacturing of soybean products (Reproduced with permission from Van Eys et al., 2004)

MATERIALS AND METHODS

Three samples of each analyzed product: whole soybean seed, soybean oil, soybean cake and full-fat soybean grits, commercially available on the market, were analyzed in aim to determine complete fatty acid profiles of soybean products. Fats were extracted by using supercritical fluid extraction (SFE) with CO2, and further analysis were done by gas chromatograph equipped with flame ionization detector (GC-FID).

Supercritical fluid extractions

LECO TFE-2000 fat analyzer (LECO, St. Joseph, MI, USA) was used for SFE, using CO2 with a purity of 99.995%. Temperature, extraction flow rates and pressure were adopted from existing LECO procedures. Cell temperature and heated variable restrictor (HVR) temperature were set at 100 ºC and 110 ºC, respectively. The collection vials on the instrument remained near room temperature of 25 ºC. Extracting pressure was 7500 psi, and extraction flow rate was 1.3 l/min. (Organic application note, 2003). “Leco Dry” infusorial soil was used as absorbent for removing traces of water from samples in amount of 2 g per 1 g of sample. Static extraction time was set on 0 minute, and dynamic extraction time was set on 60 minutes.

1.0 g of homogenized milled sample was weighed into glass baker with accuracy of ±0.001 g. Targeted mass of absorbent was added to the baker and the sample was vigorously dispersed with a glass rod. This way prepared mixture was transferred into a metal extraction thimble (12 cm length and 10 mm diameter).

Filled extraction thimbles were closed with approximately 0.5 g of glass wool on the top and appropriate cap. Glass scintillation vials (Wheaton, Millville, NJ, USA) were used as vessels for collecting extracted fat.

Prepared thimbles and collection vials were placed in the instrument. After finishing the extraction step, the instrument was depressurized, and the collection vials were removed from the instrument. Next step was degassing of extracted fat in collection vials for ten minutes, and achieving constant weight of the extract. Fat content was expressed as percent by weight.

Fatty acid determination

Fatty acid methyl esters were prepared from the extracted lipids with method that use boron trifluoride/methanol solution, as recommended method for this type of substrates (Karlović & Andrić, 1996). Nitrogen gas was used for drying and removing solvents from fatty acid methyl esters. Obtained samples were analyzed by a GC Agilent 7890A system with FID, autoinjection module for liquid, equipped with fused silica capillary column (DB-WAX 30 m, 0.25 mm, 0.50 um). Helium was used as a carrier gas (purity > 99.9997 vol %, flow rate = 1.26 ml/min). The fatty acids peaks were identified by comparison of retention times with retention times of standards from Supelco 37 component fatty acid methyl ester mix and with data from internal data library, based on previous experiments. Results were expressed as mass of fatty acid or fatty acid group (g) in 100 g of fatty acids.

Results and Discussion

The results of fatty acid analysis by GC-FID are shown in table 1. As it can be seen from the results, most prevalent fatty acids in soybean and soybean products, except in soybean oil, are C18:2 ω-6, C18:1, and C18:3 ω-3, respectively. These results are expected and in consistence with other literature data (Sauvant et al., 2004, Karlović & Andrić, 1996). In soybean oil, there has been higher concentration of C16:0 palmitic fatty acid, which is also common, and it could be consequence of soybean oil production process.

SFAs have been generally labeled as the cause of cancers and coronary heart disease. The mean ratio of PUFA/SFA recommended by the British Department of Health is more than 0.45, and WHO/FAO experts have reported guidelines for a “balanced diet” in which suggested ratio of PUFA/SFA is above 0.4 (Wood et al., 2008; Wood et al., 2003; Cardiovascular Review Group Committee on Medical Aspects of Food Policy, 1994). From this aspect, all examined samples have had favorable (from 2.49 to 12.02) PUFA/SFA ratio (Figure 2). It has been estimated that the present Western diet is deficient in ω-3 fatty acids, with a ratio of ω-6 to ω-3 of 15-20/1, instead of 1/1 as is the case with wild animals and presumably human beings (Simopoulos, 2008). Nutritional advice for today’s ω-6/ω-3 ratio is less than 4 (Scollan et al., 2006).

Table 1.

Fatty acid composition (% w/w) of soybean and various soybean products

| Fatty acid | Products |  |  |  |
| --- | --- | --- | --- | --- |
|  | Soybean | Soybean oil | Soybean cake | Full-fat soybean grit |
| C10:0 | ND | ND | 0.17 ± 0.15 | 0.10 ± 0.06 |
| C14:0 | ND | ND | 0.04 ± 0.36 | 0.05 ± 0.31 |
| C16:0 | 3.28 ± 0.12 | 16.95 ± 0.09 | 3.07 ± 0.16 | 3.11 ± 0.11 |
| C18:0 | 2.34 ± 0.23 | 5.15 ± 0.32 | 2.41 ± 0.08 | 2.53 ± 0.23 |
| C18:1 | 20.47 ± 0.51 | 16.02 ± 0.21 | 20.90 ± 0.23 | 22.22 ± 0.29 |
| C18:2 ω-6 | 68.02 ±0.06 | 47.57 ± 0.15 | 67.08 ± 0.54 | 65.82 ± 0.39 |
| C18:3 ω-3 | 5.18 ± 0.13 | 12.11 ± 0.17 | 5.78 ± 0.33 | 5.57 ± 0.15 |
| C20:0 | 0.71 ± 0.4 | 1.40 ± 0.3 | 0.20 ± 0.63 | 0.21 ± 0.35 |
| C20:1 | ND | 0.42 ± 0.41 | 0.18 ± 0.45 | 0.20 ± 0.12 |
| C22:0 | ND | 0.39 ± 0.18 | 0.17 ± 0.22 | 0.18 ± 0.41 |
| SFA | 6.33 | 23.89 | 6.06 | 6.18 |
| MUFA | 20.47 | 16.44 | 21.08 | 22.42 |
| PUFA | 73.2 | 59.68 | 72.86 | 71.39 |
| MUFA/SFA | 3.23 | 0.69 | 3.48 | 1.09 |
| PUFA/SFA | 11.56 | 2.49 | 12.02 | 11.55 |
| ∑ ω -6 | 68.02 | 47.57 | 67.08 | 65.82 |
| ∑ ω -3 | 5.18 | 12.11 | 5.78 | 5.57 |
| ω-6/ ω -3 | 13.13 | 3.92 | 11.61 | 11.82 |

Results are given as mean ± standard deviation (n = 3); SFA - saturated fatty acid;

ND – not detected; MUFA - monounsaturated fatty acids; PUFA – polyunsaturated fatty acids

Figure 2. The PUFA/SFA ratio in the examined samples

The examined soybean and soybean products had ω–6/ω–3 ratio higher than 4, as it can be noticed on Figure 3, except for the soybean oil (3.92). Lower ω-6/ω-3 ratio in soybean oil is not the rule in practice, and it could be result of different soybean type, in this particular case. Therefore, soybean cannot be used as the only source of fatty acids in human nutrition (probably also in animal nutrition), and have to be combined with other food, rich in ω-3 fatty acids.

Figure 3. The ratio of ω–6/ω–3 fatty acids in the examined samples

Conclusions

If we look at fatty acid composition, they are partially suitable for food and feed usage and cannot be used as the only source of fat in the diet. Whole soy bean, as well as soybean oil, soybean cake, and soybean grits have had PUFA/SFA ratio higher than 0.4 (from 2.49 to 12.02), which is the minimum ratio recommended from WHO/FAO organization. On the other side, ω–6/ω–3 ratio was higher than 4, and it is limitation factor for independent usage from other fat source. Knowing this and taking it into account soybean and its products have an important role in today’s human and animal nutrition.

AUTHOR CONTRIBUTIONS

Conceptualization, J.D.L.; Methodology, J.D.L. and S.A.S.; Investigation, formal analysis, validation, writing-original draft preparation, D.S.I.; Writing-review and editing, S.A.S.; Supervision, J.D.L.

DATA AVAILABILITY STATEMENT

Data contained within the article.

АCKNOWLEDGEMENTS

This paper is a result of the research within the project III046012 “Istraživanje savremenih biotehnoloških postupaka u proizvodnji hrane za životinje u cilju povećanja konkurentnosti, kvaliteta i bezbednosti hrane za životinje (Study of modern biotechnological methods in the production of animal feed in order to increase competitiveness, quality and safety of the feed)”, financed by the Ministry of Science and Technological Development, Serbia.

CONFLICT OF INTEREST

The authors declare no conflict of interest. The funders had no role in the design of the study; in the collection, analyses, or interpretation of data; in the writing of the manuscript, or in the decision to publish the results.

REFERENCES

Connor, W. E. (2000). Importance of n-fatty acids in health and disease. American Journal of Clinical Nutrition, 71, 171S- 175S. https://doi.org/10.1093/ajcn/71.1.171S

Enser, M., Richardson, R. I., Wood, J. D., Gill, B. P., & Sheard, P. R. (2000). Feeding linseed to increase the the n-3 PUFA of pork: fatty acid composition of muscle, adipose tissue, liver and sausages. Meat Science, 55, 201-212. https://doi.org/10.1016/S0309-1740(99)00144-8

Givens, D. I., Kliem, K. E., & Gibbs, R. A. (2006). The role of meat as a source of n-3 polyunsaturated fatty acids in the human diet. Meat Science, 74, 209-218. https://doi.org/10.1016/j.meatsci.2006.04.008

Cardiovascular Review Group Committee on Medical Aspects of Food Policy. (1994). Nutritional aspects of cardiovascular disease Report. Reports on Health and Social Subjects, 46, 1–186.

Karlović, Đ., & Andrić, N. (1996). Kontrola kvaliteta semena uljarica. Beograd, SRJ: Savezno ministarstvo za nauku tehnologiju i razvoj, Savezni zavod za standardizaciju.

Organic application note. (2003). LECO Corporation, St. Joseph, MI.

http://www.leco.com/resources/application_notes/pdf/TFE2000_SOYBEANS_203-821-136.pdf

Pravilnik o kvalitetu hrane za životinje. (2009). Službeni glasnik RS, 41/09.

Rose, D. P., & Connolly, J. M. (1999). Omega-3 fatty acids as cancer chemopreventive agents. Pharmacology and Therapeutics, 83, 217-244.

Sauvant, D., Perez, J.-M., & Tran, G. (Eds.). (2004). Tables of composition and nutritional value of feed materials. France:

INRA.

Scollan, N. D., Hocquette, J.-F.,  Nuernberg, K.,  Dannenberger, D., Richardson, R. I., & Maloney, A. (2006). Innovations in beef production systems that enhance the nutritional and health value of beef lipids and their relationship with meat quality. Meat Science, 74, 17-33.

https://doi.org/10.1016/j.meatsci.2006.05.002

Simopoulos, A. P. (2002).The importance of the ratio of omega-6/omega-3 essential fatty acids. Biomedicine and Pharmacotherapy, 56, 365-379. https://doi.org/10.1016/s0753-3322(02)00253-6

Simopoulos, A. P. (2008). The importance of the omega 6/omega 3 fatty acid ratio in cardiovascular disease and other chronic diseases. Experimental Biology and Medicine (Maywood), 233(6), 674-88. https://doi.org/10.3181/0711-MR-311

Torbica, A., Hadnađev, M., Dokić, P., & Sakač, M. (2008). Mixolab profiles of gluten-free products ingredients. Food Processing, Quality and Safety, 35(1), 19-26.

Van Eys, J. E., Offner, A., & Bach, A. (2004). Manual of quality analyses for soybean products in the feed industry. American Soybean Association United Soybean Board, USA.

Wood, J. D., Enser, M., Fisher, A. V., Nute, G. R., Sheard, P. R, Richardson, R. I., Huges, S. I., & Whittington, F. M. (2008). Fat deposition, fatty acid composition and meat quality: A review. Meat Science, 78, 343 – 358.

https://doi.org/10.1016/j.meatsci.2007.07.019

Wood, J. D., Richardson, R. I., Nute, G. R., Fisher, A. V., Campo, M. M., Kasapidou, E., Sheard, P. R., & Enser, M. (2003). Effect of fatty acids on meat quality: A review. Meat Science, 66, 21 – 32. https://doi.org/10.1016/S0309-1740(03)00022-6.

SASTAV MASNIH KISELINA RAZLIČITIH PROIZVODA OD SOJE

Dušica S. Ivanov*1, Jovanka D. Lević1, Slavica A. Sredanović1

1Univerzitet u Novom Sadu, Naučni institut za prehrambene tehnologije u Novom Sadu,

21000 Novi Sad, Bulevar cara Lazara br. 1, Srbija

Sažetak: U protekloj deceniji, upotreba soje i sojinih proizvoda konstantno je rasla, kako u ishrani ljudi, tako i u ishrani životinja. Danas je poznat čitav niz proizvodnih procesa, koji se primenjuju u preradi sojinog zrna radi dobijanja različitih proizvoda, koji se upotrebljavaju u prehrambenoj industriji i industriji hrane za životinje. Cilj ovog rada bio je da se ispita masnokiselinski sastav različitih proizvoda od soje, kako bi se stekla slika o njihovoj nutritivnoj podobnosti, s aspekta sadržaja masnih kiselina. Ispitivani su celo zrno soje, sojino ulje, sojina pogača i sojin griz. Ukupne masti su ekstrahovane iz uzoraka primenom superkritične fluidne ekstrakcije sa SO2, a masnokiselinski profil je određen na gasnom hromatografu sa plameno - jonizujućim detektorom. Najzastupljenije masne kiseline u svim uzorcima, osim u sojinom ulju su C18:2ω-6, C18:1 i C18:3ω-3. Svi uzorci imali su odnos PUFA/SFA veći od 0,4, što se smatra poželjnim za prehrambene proizvode. Odnos ω–6/ω-3 masnih kiselina u uzorcima bio je veći od 4 (preporučljivo je da ovaj odnos ima vrednosti manje od 4), pa se stoga soja, kao ni proizvodi od soje ne mogu koristiti kao jedini izvor masnih kiselina u ishrani ljudi i životinja. Ključne reči: masne kiseline, soja, proizvodi od soje

| Received: 25 Маy 2020/ Received in revised form: 19 August 2020/ Accepted: 28 September 2020 Available online: ????? |  |
| --- | --- |
|  | This open-access article is licensed under the Creative Commons Attribution 4.0 International License. To view a copy of this license, visit https://creativecommons.org/licenses/by/4.0/or send a letter to Creative Commons, PO Box 1866, Mountain View, CA 94042, USA. |